# Practical 2 – Interaction Description

## What visual input does the system use?

The system use a simple USB webcam to capture coloured video. The primary focus of the visual input is the identification of static hand gestures within the frame. The model used in code was trained in google teachable and outputted as an LiteRT or TensorFlow Lite floating point.

## How is this input interpreted?

The interpretation is handled through a Computer Vision (CV) pipeline using a pre-trained LiteRT model (model_unquant.tflite).

- Feature Extraction: The system processes the image to identify patterns matching the trained gesture classes.

- Classification: The np.argmax function identifies the gesture with the highest probability.

- Thresholding: The system only acts if the model’s confidence exceeds a 90% confidence threshold. This prevents unintended inputs from being sent to the system.

## System Behaviour

The system acts as a virtual HID (Human Interface Device) bridge. When a specific gesture is recognized with high confidence, the system maps that gesture to a specific keyboard output (WASD) and sends it to the Raspberry Pi's operating system using the keyboard library.

- Continuous Input: Unlike a single keypress, the system maintains the keyboard.press() state as long as the gesture is held, releasing it only when the gesture changes or confidence drops.

- State Toggle: A global toggle (via the Spacebar) allows the user to enable or disable gesture-to-keyboard mapping without closing the visual feed.

## UX Decisions

- Real-time Feedback Loop: The system overlays the detected label and the exact confidence percentage directly onto the camera feed. This allows the user to understand "why" the system is or isn't responding (e.g., if they need to move their hand closer or improve lighting).

- Visual Status Indicators: The UI uses color-coded status text (Green for ACTIVE, Red for PAUSED) to provide immediate visibility of the system state.

- Accessibility through Simplicity: The choice of generic directional gestures (Up, Down, Left, Right) ensures the system is intuitive and requires no complex manual to operate.

## Intended Context or User

The system is designed for gamers or hobbyists interested in alternative input methods. It is particularly suited for:

- Users looking for a "hands-free" or novel way to interact with simple arcade-style games.

- Educational demonstrations of Edge AI and gesture recognition on low-power hardware like the Raspberry Pi.

## How does the system show it is “watching”, unsure, or confident?

The system communicates its internal state through a dynamic Heads-Up Display (HUD):

- Watching: Indicated by the live video window and the "Status" label.

- Unsure: If the confidence value displayed on the screen is below $90\%$, the system stops all keyboard outputs. The user sees the dropping percentage on the HUD, providing a clear visual cue that the gesture is not distinct enough.

- Confident: When confidence exceeds $90\%$, the label becomes stable on the screen, and the corresponding game action (movement) occurs, providing immediate haptic/visual feedback within the game.