# Practical 3 – Interaction Description

# What input does the system use?

The system uses 2 sensors, the Accelerometer & Gyroscope Module (MPU6050) which detects rotation, acceleration and temperature, and the Pulse Oximeter and Heart Rate Sensor (MAX30102), which measures heartrate and blood oxygen levels.

# System Behaviour

Both sensors' values are read in the practical3.py script file. The data is then sent through a WebSocket to the script.js then values are then interrupted and displayed on the local server the .py file opened.

Python Script: Raspberry Pi doesn’t have a python module for the MAX30102, but I found a git repo online and clone that into the project https://github.com/doug-burrell/max30102. Once I import the module for the repository, reading that values was straight forward.

I decided to run the 2 sensors on different threads as it was the easiest way to get constant updates from the Accelerometer & Gyroscope Module without being interrupted by the long wait time on the heartrate sensor, need to use threading. Lock() to make sure emits aren’t sent at the same time corrupting data. Values are sent as they are received.

Acceleration: (x,y,z) values are tracked and displayed in a stack from the screens centre, all values are flipped positive for the visualisation. As values come in they are stored in a data stream array, the values in this array are averaged as new values arrive. After 200ms the average value is committed, and a new stack of values is rendered 15 deg anticlockwise.

Rotation: rotation values are displayed in mostly the same way, just being offset from by 7.5 degs from the acceleration stack. And a different multiplier is used to scale the length of the values.

Heart Rate: the sensors send the BPM values to the script. The centre white circle is scaled based on the bpm and the exact value is displayed in white in the centre. If the BPM values is invalid instead a message telling the user to adjust their finger and make sure its properly placed.

Keys: in the right corner the recieved data is displayed along with a label and a corresponding colour.

Effects: old Accelerometer & Gyroscope values are faded out, after about 30 stored values. The array storing previous readings is shifted, deleting the first value.

# Why did you choose this visual form?

The purpose of reading accel, gyro and HR is to show how the users heart rate maybe change through movement (Acceleration/Rotation) of the sensor. Like some sort of fitness tracker/visualiser.

I chose to represent this data in a circular bar chart format, as it shows clearly the passage of time and which value are most recent, through a clock or stopwatch metaphor. Which also match's the fitness theme where users often want to see progression through time and often use a stopwatch as a start or stop point for an exercise.

Having the heart BPM in the centre of this activity adds to this effect, showing clearly the purpose of the program. The white circle also moves based on bpm which helps show a clear connection between movement and heart rate.

# Intended Context or User?

As state before this program is intended for any user wanting to track how activity effects their heart rate. From this data they can interrupt their fitness level.

One Hardware limitation is that the heart rate sensor needs to be in a stable consistent location on the user, ideally where their skin isn’t to think so the reader can get a proper reading. Which can get in the way when wanting to move around or exercise.

That issue lowers the contexts where the system works, so ideally you'd be measuring some sort of hand or arm exercise .

# Data & Visual Mapping Specification

| Data Source | Data Type | Interpretation Method | Visual Variable |
| --- | --- | --- | --- |
| MPU6050 (Accel) | Continuous (X, Y, Z) | 200ms Smoothing (Averaging) | Line Length & Position (Radial) |
| MPU6050 (Gyro) | Continuous (X, Y, Z) | 200ms Smoothing (Averaging) | 7.5° Angular Offset from Accel |
| MPU6050 (Temp) | Continuous (Celsius) | Displayed on arrival | Text key on right side of screen |
| MAX30102 (HR) | Discrete (BPM) | Thresholding (0–300 Range Check) | Circle Scale (Radius) |
| Time | Temporal | Accumulation (Array of 30) | Rotation (Angle) & Opacity (Fade) |
| Sensor Error | Classification | Boolean Check (Valid/Invalid) | Text Warning (Centre Screen) |