# Interaction Description

# Sensor

Our practical use the joystick module to sense user data, it has an analog output that needs to be converted by the PCF8591 module, this then connects to the raspberry pie. This system outputs the x and y position of the joystick, and also if its pressed in. Sound is played through the passive buzzer module

# Interaction

Our script then reads these x/y values and changes the LCD, moving the stick up and down changes the selected song, moving the stick left starts the song, and holding it right ends the song, the state and index of songs in handled by this system.

Need to hold right on the stick to turn the song off is necessary because of how the system polls during song playback, I could find a simple way around this issue without chopping up the note into many small duration and polling between them, which I thought was a bit too complex for this first practical, UX drawbacks of holding the stick right were also minor.

# Output

In the main loop, when the user plays a song, the “play selected” function runs, this loops through the notes in the selected song and tells the buzzer to play a tone for duration. Based on the song data, songs are comprised of a list of tuples,

Note Tuple: first position contains note pitch, written as a string “C4”  being the middle C of a piano. In the second position the note duration, how  long the program sleeps before playing a new pitch.

The first song “pink panther” was taken from the passive buzzer tutorial page, , rest I made myself.

To emulate the sound of chords I created a function inside of songLibrary.py, this function takes any number of notes as an argument (inside a list), these notes will make up the chord outputted, then a duration of the chord. Then a for loops goes through the note giving them a tiny duration. And pushing them back to a list (same format as song data), the final output is a long list of notes playing rapidly after each other. The final effect is a little distorted and buzzy, but you can make out continuous chords.

This was used for 2 of the songs I created.

# UX

There was a few UX considerations made, the main one being the use of a joystick to control the system, the joystick likly being the most intuitive and ergonomic module for user input in the kit and allowing for many different inputs if needed. It came with the drawback of needing extra setup and wire mess because of the analog converter (PCF8591), but was worth the extra effort to create an intuitive comfortable experience.

Another thing to consider was how the user would figure out the navigation. Since the LCD could display 2 lines at once I decide to reserve the top line as controls list, it was hard to compact the information into such a small space so I had to use symbols and shortened words "^v:sel <>:on/off".

When a direction is inputted, there is a short delay before another input can be registered this is to stop the user for scrolling more than they want, making song selection a lot easier.

# Sensor & Input Specification

- PCF8591

  - analog-to-digital/digital-to-analog converter (ADC DAC)

  - Reads continuous data

  - Only takes input from the joystick so no environmental factors would affect it

- Passive buzzer

  - Referred to as TonalBuzzer

  - N/A

  - N/A

- Joystick

  - Called “button” in the code

  - Reads continuous data

  - No environmental factors affect the joystick

- LCD

  - Called lcd in code

  - N/A

  - N/A